REPUBLIC OF YEMEN

SANA'A UNIVERSITY

FACULTY OF ENGINEERING

MECHATRONICS ENGINEERING DEPARTMENT

ذ

PLC TAGGING BASED ON COLOR

Done by:

Hassan Moqbel Morshed 202274198

Musa AL_salmani 202174202

Systems (parallel).

Supervised by:

Dr. Marowan Noaman

ENG. Mohammed Al-zaqheer

Contents

Introduction	3

Objectives	3

Theory	3

System Analysis	3

System Requirements:	4

Flowchart	5

Simulation	6

Discussion	6

Conclusion and recommendations	6

# Introduction

This project represents a practical application for automating the process of sorting products by color in industrial environments. The system is based on a 1200-PLC S7 module with an advanced vision system to automatically identify and sort products. The system provides an effective solution for factories that require accurate and fast sorting of products.

# Objectives

- Design of a high-precision automatic sorting system for products by color.

- Achieve a processing rate of 30 pieces/minute.

- Provide an easy-to-use control interface.

- Ensure the reliability and security of the process.

- Reducing human intervention in the sorting process

# Theory

The system is based on the following principles:

- Machine vision technology for color detection.

- Programmable Logic Control (PLC)

- Mechanical transmission systems.

- Pneumatic control systems

- HMI integration with PLC.

# System Analysis

1. Description of the process:

- Product entry on the main belt.

- Check the product color by vision sensor.

- Directing green products to the secondary belt.

- Blue products continue on the main belt.

- Assemble and deliver products to their final locations.

2. Main components:

- Main conveyor belt

- Secondary conveyor belt for green products

- Vision Sensor

- Pneumatic Pusher

- Two-Axis Pick & Place Robotic Arm

- Photoelectric sensors

- electrical control panel.

3. Technical Specifications:

- Console

- PLC: Siemens S7-1200 (CPU 1211C DC/ DC/DC)

- Working memory: 50 KB

- Power supply: 24VDC

- Communication ports: PROFINET

- HMI screen

- Comfort Type TP700

- Screen resolution: 800 x 480 pixels

- PROFINET: Connection

4. Sensors and Actuators Analysis:

i.  Sensors

- Automatic vision sensor

- Type RGB Color Sensor

- Accuracy  0.1mm

- Response Time: 1ms

- Operating voltage: 24VDC

2. Photoelectric sensors

- Type Diffuse

- Detection range(100mm-10)

- Operating voltage: 24VDC

- Sensors(4)

ii.  Triggers

- Belt drives

Type is a DC Geared Motor

Capacity (0.5HP)

Speed (1440RPM)

Number (2)

2.  air motive

Operating pressure: 6 bar

Thrust force: 100N

Response time: 50ms

iii. Estimated cost

- PLC S7-1200: $800

- HMI TP700: $2,000

- Vision sensor $1,500

- Conveyor belts: $3,000

- Air Motivator: $500

- Robotic arm: $4,000

- Photoelectric sensors: $400

- Control panel and cables $800

- Estimated Total: $13,000

# System Requirements:

- Digital Inputs:

Start_PB: (I0.0) Start button

Stop_PB:   (I0.1) Stop button

Emergency_Stop:  (I0.2) Emergency button

Vision_Sensor:  (I0.3) Vision sensor

Conv1_Sensor1:  ( I0.4) Main belt sensor 1

Conv1_Sensor2:  (I0.5)  Main belt sensor 2

Conv2_Sensor1:  (I0.6) Secondary belt sensor 1

Conv2_Sensor2:  ( I0.7) Secondary belt sensor 2

Pick_Place_Home:  ( I1.0) Main arm position

Pick_Place_End:   ( I1.1) End arm position

- Digital Outputs:

Main_Conv:   (Q0.0) Main belt operation

Secondary_Conv:  (Q0.1) Secondary belt operation

Pusher_Forward: (Q0.2) Pusher advance

Pusher_Backward:  (Q0.3) Pusher return

Pick_X_Forward:  (Q0.4) X forward movement

Pick_X_Backward:  (Q0.5) X backward movement

Pick_Y_Up:   (Q0.6) Y up movement

Pick_Y_Down:  (Q0.7) Y down movement

- Counters and Timers:

Blue_Counter: C1

Green_Counter: C2

Pusher_Timer: T1

Pick_Place_Timer: T2

# Flowchart

# Simulation

- For software simulation 1 PLCSIM

- Factory I/O For 3D simulation

- WinCC Runtime for HMI simulation

# Discussion

1. HMI and Factory I/O Connectivity Issues

- Problem: HMI not connecting with Factory I/O.

- Causes:

- Mismatched tag addresses.

- Incorrect TIA Portal settings.

- Licensing issues.

- Solutions:

- Standardized tag names.

- Reconfigured network settings.

- Updated software licenses.

2. System Response Delays

- Problem: Delays in actuator responses.

- Causes:

- Long PLC scan cycles.

- Resource contention.

- Slow sensor signal processing.

- Solutions:

- Optimized algorithms.

- Reduced scan cycle time.

- Used interrupts for critical functions.

# Conclusion and recommendations

i.  Results

- Classification accuracy up to 99%.

- Processing speed: 30pcs/min.

- User-friendly control interface.

i.  Recommendations

- Add an automated reporting system

- Possibility of expansion Additional colors

- Connect the system to a central database.

- Adding an alarm system for preventive maintenance.

IMAGES

FACTORY IO IMAGE

HMI IMAGE